ISP

Hostname:

hostnamectl set-hostname isp

mcedit /etc/sysconfig/network 
HOSTNAME=isp

exec bash

iptables -t nat -A POSTROUTING -s 172.16.1.0/28 -o ens3 -j MASQUERADE

iptables -t nat -A POSTROUTING -s 172.16.2.0/28 -o ens3 -j MASQUERADE

iptables-save > /etc/sysconfig/iptables

systemctl enable --now iptables

HQ-RTR:

(база)

hostname hq-rtr

ip domain-name au-team.irpo

ntp timezone utc+3

ip route 0.0.0.0/0 172.16.1.1

(тунель гре)

interface tunnel.0

ip address 10.10.10.1/30

ip tunnel 172.16.1.2 172.16.2.2 mode gre

(ospf)

router ospf 1

ospf router-id 10.10.10.1

passive-interface default

no passive-interface tunnel.0

network 10.10.10.0/30 area 0

network 10.10.100.0/27 area 0

network 10.10.200.0/28 area 0

network 10.10.30.0/29 area 0

exit

interface tunnel.0

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 P@ssw0rd

(nat)

interface isp

ip nat outside

exit

interface vl100

ip nat inside

exit

interface vl200

ip nat inside

exit

interface vl999

ip nat inside

exit

ip nat pool VLAN100 10.10.100.1-10.10.100.30

ip nat pool VLAN200 10.10.200.1-10.10.200.14

ip nat pool VLAN999 10.10.30.1-10.10.30.6

ip nat source dynamic inside-to-outside pool VLAN100 overload interface isp

ip nat source dynamic inside-to-outside pool VLAN200 overload interface isp

ip nat source dynamic inside-to-outside pool VLAN999 overload interface isp

write memory

BR-RTR:

hostname br-rtr

ip domain-name au-team.irpo

ntp timezone utc+3

ip route 0.0.0.0/0 172.16.2.1

(тунель гре)

interface tunnel.0

ip address 10.10.10.2/30

ip tunnel 172.16.2.2 172.16.1.2 mode gre

(ospf)

router ospf 1

ospf router-id 10.10.10.2

passive-interface default

no passive-interface tunnel.0

no passive-interface fw

network 10.10.10.0/30 area 0

network 10.20.10.0/30 area 0

exit

interface tunnel.0

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 P@ssw0rd

interface fw

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 P@ssw0rd

(nat)

interface isp

ip nat outside

exit

interface fw

ip nat inside

ip nat pool VLAN10 10.20.20.1-10.20.20.14

ip nat pool VLAN20 10.20.30.1-10.20.30.6

ip nat pool RTR-FW 10.20.10.1-10.20.10.2

ip nat source dynamic inside-to-outside pool VLAN10 overload interface isp

ip nat source dynamic inside-to-outside pool VLAN20 overload interface isp

ip nat source dynamic inside-to-outside pool RTR-FW overload interface isp

exit

hq-srv:

hostnamectl set-hostname hq-srv.au-team.irpo

mcedit /etc/sysconfig/network

HOSTNAME=hq-srv.au-team.irpo

echo “10.10.100.2/27” > /etc/net/ifaces/ens3/ipv4address

echo “default via 10.10.100.1” > /etc/net/ifaces/ens3/ipv4route

echo “nameserver 10.10.100.2 77.88.8.8” > /etc/net/ifaces/ens3/resolv.conf

systemctl restart network

Дальше вам нужно искать ошибки в файлах. По очереди открываете файлы и внимательно смотрите, чтобы было как на скринах.

mcedit /etc/bind/options.conf

Следующие два файла почти идентичные, смотрите только верхнюю часть

mcedit /etc/bind/zone/10.10.in-addr.arpa

mcedit /etc/bind/zone/16.172.in-addr.arpa

Тут чуть больше, смотрите внимательно

mcedit /etc/bind/zone/au-team.irpo.zone

systemctl restart bind.service

echo "Banner /etc/openssh/banner" >> /etc/openssh/sshd_config

echo "Authorized access only" > /etc/openssh/banner

systemctl restart sshd

br-srv:

hostnamectl set-hostname br-srv.au-team.irpo

mcedit /etc/sysconfig/network

HOSTNAME=br-srv

echo “10.20.20.2/28” > /etc/net/ifaces/ens3/ipv4address

echo “default via 10.20.20.1” > /etc/net/ifaces/ens3/ipv4route

echo “nameserver 10.10.100.2 77.88.8.8” > /etc/net/ifaces/ens3/resolv.conf

systemctl restart network

echo "Banner /etc/openssh/banner" >> /etc/openssh/sshd_config

echo "Authorized access only" > /etc/openssh/banner

systemctl restart sshd

HQ-SW

hostnamectl set-hostname hq-sw.au-team.irpo

mcedit /etc/sysconfig/network

HOSTNAME=hq-sw.au-team.irpo

hq-cli:

заходите в терминал (правой кнопкой мыши по рабочему столу)

su –

hostnamectl set-hostname hq-cli.au-team.irpo

mcedit /etc/sysconfig/network

HOSTNAME=hq-cli.au-team.irpo

exec bash

br-cli:

su -

hostnamectl set-hostname br-cli.au-team.irpo

mcedit /etc/sysconfig/network

HOSTNAME=br-cli.au-team.irpo

exec bash

Теперь настроим ospf на br-fw

Правой кнопкой мыши

Заходите на проводное подключение 2 и вырубаете

Не забудьте сохранить (пароль toor)

Дальше заходим на первое подключение и вставляем ip

Далее заходим в браузер

В строку вбиваете

Вам может сказать, что сайт опасный, но на него всё же надо зайти.

Дальше будет вкладка авторизации. Вам будет выдана бумажка. Нам нужно смотреть в :”Учётные записи ВМ”, Ideco NGFW
Дальше открываете меню и пишите название.

Дальше в том же меню ищите «маршрутизация» >>  «ospf»

Делаете как я

В конце включаем эту тему

Выходим и включаем назад проводное подключение 2

!!!Если айпишники другие, то:

Заходишь в топологии на br-fw и смотришь ip (оно должно вылезти сразу)

Айпишник у коготорого третий блок будет больше всего – скорее всего является нужным :/

Так что на br-cli в строке адрес пишите его, ток в конце пишете двойку вместо единицы и всё. В шлюз пишите тот же самый ток с единицей. Шлюза всегда 29.